INSTRUCTOR ANSWER KEY · INTERNAL · DO NOT DISTRIBUTE TO STUDENTS

# LAB 03 ANSWER KEY
RV32I SINGLE-CYCLE DATAPATH

Reference answers for instruction, grading, and hidden-test construction; run the full regression on the course simulator before official release.

| 03 week | 3 experiments | 4 hidden tests | INTERNAL release level |
| --- | --- | --- | --- |

Language and sources: All solutions use Verilog-2001 (.v). The Week 02/03/04/06/07 code was adapted from the submitted RISC-V codebases; Week 01 and Week 05 were added under the same contract.

Source adjustments: Reuse the decode approach from Control_Unit.v; add safe defaults and enable RegWrite only for supported funct3 values.

# 1. Quick answer summary

| Topic | Required answer/contract |
| --- | --- |
| R-type opcode | 0110011; RegWrite=1, ALUSrc=0, ResultSrc=ALU, no memory/branch. |
| ADDI opcode | 0010011; RegWrite=1, ALUSrc=1, ImmSrc=I, ResultSrc=ALU. |
| ALU decode | funct3 selects ADD/SUB/SLT/OR/AND; SUB only when R-type and funct7[5]=1. |
| Unsafe opcode | All writes/redirects deasserted; output select has a defined value. |

# 2. Answers for each experiment

## Experiment 1: Control table

Required conclusion: Convert instruction fields into a safe control word.

### Core answer

- Use symbols/enums for ResultSrc, ImmSrc, and ALUControl to avoid dependence on implicit encodings.

- The main decoder creates the control class; the ALU decoder uses ALUOp+funct.

- Unsupported opcode/funct combinations must fall through to the safe default.

### Verilog-2001 reference solution

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_03/control_unit_alu_subset.v

module control_unit_alu_subset(
    input wire [6:0] op,
    input wire [2:0] funct3,
    input wire funct7_5,
    output reg RegWrite, ALUSrc,
    output reg [1:0] ImmSrc, ResultSrc,
    output reg [2:0] ALUControl);
    always @(*) begin
        RegWrite=0; ALUSrc=0; ImmSrc=2'b00;
        ResultSrc=2'b00; ALUControl=3'b000;
        case (op)
            7'b0110011: begin
                case (funct3)
                    3'b000: begin RegWrite=1; ALUControl=funct7_5?3'b001:3'b000; end
                    3'b111: begin RegWrite=1; ALUControl=3'b010; end
                    3'b110: begin RegWrite=1; ALUControl=3'b011; end
                    3'b010: begin RegWrite=1; ALUControl=3'b101; end
                    default: begin end
                endcase
            end
            7'b0010011: begin
                if (funct3==3'b000) begin
                    RegWrite=1; ALUSrc=1; ImmSrc=2'b00; ALUControl=3'b000;
                end
            end
            default: begin end
        endcase
    end
endmodule

### Expected results

| Scenario | Correct result |
| --- | --- |
| add/sub/and/or/slt | Write rd from the ALU |
| addi | Write rd from rs1+I-imm |
| unsupported opcode | No RegWrite/MemWrite/PC redirect |

### Errors requiring a deduction or correction

- Using a MIPS opcode.

- Detecting SUB from funct7 without confirming R-type.

- Leaving RegWrite=1 in the safe default.

Waveform oracle: The submitted image must allow direct comparison of PC, Instr, control word, RD1/RD2, SrcB, ALUResult, Result, and RegWrite; an illegal opcode must show write enable deasserted.

## Experiment 2: ALU-subset datapath

Required conclusion: Integrate IF–ID–EX–WB in one long cycle.

### Core answer

- One instruction passes through all blocks in the same cycle; state updates only at the clock edge.

- The trace must prove that rs/rd/imm/control belong to the same Instr.

- Writes to x0 remain blocked by the Register File.

### Verilog-2001 reference solution

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_03/single_cycle_alu_datapath.v

module single_cycle_alu_datapath(
    input wire clk, reset_n,
    input wire [31:0] Instr,
    output wire [31:0] PC,
    output wire [31:0] Result);
    wire [31:0] PCPlus4, ImmExt, RD1, RD2, SrcB, ALUResult;
    wire [4:0] Rs1, Rs2, Rd;
    wire RegWrite, ALUSrc, Zero;
    wire [1:0] ImmSrc, ResultSrc;
    wire [2:0] ALUControl;

    assign PCPlus4=PC+32'd4;
    assign Rs1=Instr[19:15]; assign Rs2=Instr[24:20]; assign Rd=Instr[11:7];
    assign SrcB=ALUSrc?ImmExt:RD2;
    assign Result=ALUResult;

    program_counter pc0(clk,reset_n,1'b1,PCPlus4,PC);
    control_unit_alu_subset cu(Instr[6:0],Instr[14:12],Instr[30],
        RegWrite,ALUSrc,ImmSrc,ResultSrc,ALUControl);
    register_file rf(clk,reset_n,RegWrite,Rs1,Rs2,Rd,Result,RD1,RD2);
    extend ext(ImmSrc,Instr[31:7],ImmExt);
    alu alu0(ALUControl,RD1,SrcB,ALUResult,Zero);
endmodule

### Expected results

| Scenario | Correct result |
| --- | --- |
| add x3,x1,x2 | SrcA=x1, SrcB=x2, Result=ALUResult |
| addi x4,x3,-1 | SrcB=FFFFFFFF |
| rd=0 | No architectural state change |

### Errors requiring a deduction or correction

- Connecting the rs2 and rd fields incorrectly.

- Misaligning control/data by registering one branch but not another.

- Reversing the result-mux source encoding.

Waveform oracle: The submitted image must allow direct comparison of PC, Instr, control word, RD1/RD2, SrcB, ALUResult, Result, and RegWrite; an illegal opcode must show write enable deasserted.

## Experiment 3: Directed regression

Required conclusion: Build a signature instead of checking every waveform by eye.

### Core answer

- The signature should be a final destination register or a simple hash of register/memory state.

- The canonical Week 03 test may use a sequence that produces x31 and checks its final value.

- The illegal-opcode test must prove that the PC still advances and state is not written incorrectly.

### Verilog-2001 reference solution

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_03/tb_single_cycle_signature.v

`timescale 1ns/1ps
module tb_single_cycle_signature;
    reg clk, reset_n; reg [31:0] Instr;
    wire [31:0] PC, Result;
    integer errors;
    single_cycle_alu_datapath dut(clk,reset_n,Instr,PC,Result);
    always #5 clk=~clk;
    initial begin
        clk=0; reset_n=0; Instr=32'h00000013; errors=0;
        #12 reset_n=1;
        Instr=32'h00500093; @(posedge clk); #1; // addi x1,x0,5
        Instr=32'h00700113; @(posedge clk); #1; // addi x2,x0,7
        Instr=32'h002081B3; @(posedge clk); #1; // add x3,x1,x2
        if (dut.rf.regs[3] !== 32'd12) errors=errors+1;
        Instr=32'h00000000; @(posedge clk); #1; // illegal/unsupported: no state change
        if (dut.rf.regs[3] !== 32'd12) errors=errors+1;
        if (errors==0) $display("TEST_PASS");
        else $display("TEST_FAIL errors=%0d",errors);
        $finish;
    end
    initial begin #1000; $display("TEST_TIMEOUT"); $finish; end
endmodule

### Expected results

| Scenario | Correct result |
| --- | --- |
| Directed ALU program | Golden x31 matches |
| Signed SLT -1<1 | rd=1 |
| Illegal instruction | No side effect |

### Errors requiring a deduction or correction

- Waiting indefinitely.

- Checking only the PC or a waveform.

- A program that accidentally depends on an unsupported instruction.

Waveform oracle: The submitted image must allow direct comparison of PC, Instr, control word, RD1/RD2, SrcB, ALUResult, Result, and RegWrite; an illegal opcode must show write enable deasserted.

# 3. Recommended hidden tests

| Test | Stimulus/intent | Oracle |
| --- | --- | --- |
| R funct collision | Same funct3=000, change funct7[5]. | ADD/SUB are distinguished correctly. |
| Illegal side effect | Insert an unsupported opcode between two valid instructions. | PC continues; RF/memory do not change except for valid instructions. |
| Signed SLT chain | The slt result is used immediately by add. | Final value matches. |
| x0 destination | Every ALU type writes rd=x0. | x0 does not change. |

Principle: A hidden test must check functionality and side effects, include a timeout, and not depend on internal hierarchy unless the published interface requires it.

# 4. Answers to oral examination questions

## 1. Which instruction creates the longest critical path?

In Week 03, the long path runs from the PC through instruction memory, decode/register read, the ALU, and the result mux to Register File setup; whether R-type or addi is longer depends on the implementation. When memory is added in Week 04, lw is often the longest-path candidate. The conclusion must come from the timing report, not from guessing by instruction name.

## 2. Which side effect can an incorrect default control create?

RegWrite or MemWrite asserted for an unsupported opcode can corrupt registers/memory; incorrect Branch/Jump can change the PC; incorrect ResultSrc/ALUSrc can write garbage data. A safe default must disable every write and redirect.

## 3. Why is signed slt different from an unsigned comparison?

With the same bit pattern, a value with MSB=1 is negative when signed but large when unsigned. For example, 0xFFFFFFFF < 1 is true when signed (-1<1) but false when unsigned.

Student submission package reference: The main report is the completed student manual exported as DSD_Labxx_MSSV_HoVaTen.pdf. It must contain the student information, RTL code, testbench code, and waveform evidence embedded after each experiment. The package must also include clean rtl/single_cycle/ and sim/single_cycle/ directories, control_table.pdf, trace_one_instruction.pdf, regression.log, and rtl_schematic.png.

# 5. Grading gates and minimum evidence

| Gate | Instructor verification method |
| --- | --- |
| Six ALU/addi instructions pass the directed tests. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Control defaults create no side effect. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Signature test PASS trong timeout. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| The critical path is explained from the PC to Register File write data. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |

# 6. Unified grading rules

| Item | Weight | Answer-key guidance |
| --- | --- | --- |
| Functionally correct RTL | 40% | Pass public and hidden tests; ISA or side-effect errors are severe. |
| Verification | 25% | The test has an oracle, corner cases, a timeout, and a reproducible log. |
| Architecture explanation | 20% | Explain the cause, signal lifetime, and trade-offs; do not merely read the waveform. |
| Interface engineering | 10% | Correct width, names, reset polarity, clock edge, and module contract. |
| Clean submission package | 5% | Runs from a clean folder; no db/output_files/work/WLF. |

Not a golden implementation: If the starter RTL, hex program, or control encoding changes, increment the answer-key version and rerun the full regression before using it for grading.