INSTRUCTOR ANSWER KEY · INTERNAL · DO NOT DISTRIBUTE TO STUDENTS

# LAB 04 ANSWER KEY
SINGLE-CYCLE MEMORY, BRANCH AND FPGA

Reference answers for instruction, grading, and hidden-test construction; run the full regression on the course simulator before official release.

| 04 week | 4 experiments | 4 hidden tests | INTERNAL release level |
| --- | --- | --- | --- |

Language and sources: All solutions use Verilog-2001 (.v). The Week 02/03/04/06/07 code was adapted from the submitted RISC-V codebases; Week 01 and Week 05 were added under the same contract.

Source adjustments: Reuse Data_Memory.v and a separate target adder; add a range guard, the full baseline core, and explicit BranchTakenE/PCSrcE separation.

Not a golden implementation: If the starter RTL, hex program, or control encoding changes, increment the answer-key version and rerun the full regression before using it for grading.

# 1. Quick answers

| Topic | Required answer/contract |
| --- | --- |
| Data memory | Combinational read, posedge write when MemWrite=1; word index from the ALU byte address. |
| Branch | PCTarget=PC+ImmExt; BranchTaken=Branch&Zero; PCNext selects target/PC+4. |
| Baseline | add, sub, and, or, slt, addi, lw, sw, beq must produce a consistent signature. |
| FPGA | The wrapper does not change the core; device 5CSXFC6D6F31C6, CLOCK_50=20 ns, QSF loads the SDC. |

# 2. Answers by experiment

## Experiment 1: Load/store

Required conclusion: Preserve alignment and write-enable behavior.

### Core answer

- ALUResult is a byte address; a word-array RAM uses ALUResult[DMEM_AW+1:2].

- WriteData comes from rs2; write at posedge when MemWrite=1.

- ReadData is meaningful only for a valid address and must be combinationally stable.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_04/data_memory.v

module data_memory #(parameter WORDS=64)(
    input wire clk, MemWrite,
    input wire [31:0] addr, write_data,
    output wire [31:0] read_data);
    reg [31:0] RAM [0:WORDS-1];
    integer i;
    initial for (i=0;i<WORDS;i=i+1) RAM[i]=32'd0;
    assign read_data=(addr[31:2]<WORDS)?RAM[addr[31:2]]:32'd0;
    always @(posedge clk)
        if (MemWrite && (addr[31:2]<WORDS)) RAM[addr[31:2]]<=write_data;
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| sw 12,0(x0) | ram[0]=12 after the edge |
| lw ...,0(x0) | ReadData=12 |
| MemWrite=0 | RAM unchanged |
| addr=4 | ram[1] |

### Errors requiring deduction or correction

- Use ram[addr].

- Combinational RAM write.

- The store uses the immediate or a forwarded ALU operand instead of rs2 data.

Waveform oracle: The submitted image must allow direct comparison of PC, PCTarget, Branch, Zero, BranchTaken/PCSrc, ALUResult, MemWrite, address, WriteData, and ReadData; distinguish taken from not-taken.

## Experiment 2: Branch on the EX-equivalent path

Required conclusion: In the single-cycle design, decide the PC using compare and an independent target adder.

### Core answer

- The B-immediate already has bit 0=0; do not shift again if Extend has already created the complete byte offset.

- Taken selects PC+ImmExt; not-taken selects PC+4.

- A backward branch requires correct sign extension.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_04/branch_at_ex_path.v

module branch_at_ex_path(
    input wire [31:0] PCE, ImmExtE,
    input wire BranchE, ZeroE, JumpE,
    output wire [31:0] PCTargetE,
    output wire BranchTakenE, PCSrcE);
    assign PCTargetE=PCE+ImmExtE;
    assign BranchTakenE=BranchE&ZeroE;
    assign PCSrcE=BranchTakenE|JumpE;
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| beq equal,+8 | PCNext=PC+8 |
| beq not equal,+8 | PCNext=PC+4 |
| beq equal,-8 | PCNext=PC-8 |

### Errors requiring deduction or correction

- Compute the target from PC+4.

- Shift the B immediate twice.

- The branch checks only Zero and forgets the Branch enable.

Waveform oracle: The submitted image must allow direct comparison of PC, PCTarget, Branch, Zero, BranchTaken/PCSrc, ALUResult, MemWrite, address, WriteData, and ReadData; distinguish taken from not-taken.

## Experiment 3: Full baseline regression

Required conclusion: Run the 9 instructions in a program with a loop, memory, and a signature.

### Core answer

- One canonical regression uses: addi x1,5; addi x2,7; add x3,x1,x2; sw x3,0(x0); lw x4,0(x0); beq x3,x4,+8; addi x5,99; sub x6,x4,x1; and x7,x3,x6; or x8,x1,x2; slt x9,x1,x2.

- The branch must skip the instruction that writes x5.

- The final result is used as the golden architectural signature.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_04/single_cycle_core.v

module control_unit_full(
    input wire [6:0] op,
    input wire [2:0] funct3,
    input wire funct7_5,
    output reg RegWrite, ALUSrc, MemWrite, Branch,
    output reg [1:0] ImmSrc, ResultSrc,
    output reg [2:0] ALUControl);
    always @(*) begin
        RegWrite=0; ALUSrc=0; MemWrite=0; Branch=0;
        ImmSrc=2'b00; ResultSrc=2'b00; ALUControl=3'b000;
        case (op)
          7'b0110011: begin
            case (funct3)
              3'b000: begin RegWrite=1; ALUControl=funct7_5?3'b001:3'b000; end
              3'b111: begin RegWrite=1; ALUControl=3'b010; end
              3'b110: begin RegWrite=1; ALUControl=3'b011; end
              3'b010: begin RegWrite=1; ALUControl=3'b101; end
              default: begin end
            endcase
          end
          7'b0010011: if (funct3==3'b000) begin
            RegWrite=1; ALUSrc=1; ImmSrc=2'b00; ALUControl=3'b000;
          end
          7'b0000011: if (funct3==3'b010) begin
            RegWrite=1; ALUSrc=1; ImmSrc=2'b00;
            ResultSrc=2'b01; ALUControl=3'b000;
          end
          7'b0100011: if (funct3==3'b010) begin
            MemWrite=1; ALUSrc=1; ImmSrc=2'b01; ALUControl=3'b000;
          end
          7'b1100011: if (funct3==3'b000) begin
            Branch=1; ImmSrc=2'b10; ALUControl=3'b001;
          end
          default: begin end
        endcase
    end
endmodule

module single_cycle_core(
    input wire clk, reset_n,
    input wire [31:0] Instr, ReadData,
    output wire [31:0] PC, ALUResult, WriteData, Result,
    output wire MemWrite);
    wire [31:0] PCPlus4, PCTarget, PCNext, ImmExt, RD1, RD2, SrcB;
    wire [1:0] ImmSrc, ResultSrc;
    wire [2:0] ALUControl;
    wire RegWrite, ALUSrc, Branch, Zero, PCSrc;

    assign PCPlus4=PC+32'd4;
    assign PCTarget=PC+ImmExt;
    assign PCSrc=Branch&Zero;
    assign PCNext=PCSrc?PCTarget:PCPlus4;
    assign SrcB=ALUSrc?ImmExt:RD2;
    assign WriteData=RD2;
    assign Result=(ResultSrc==2'b01)?ReadData:
                  (ResultSrc==2'b10)?PCPlus4:ALUResult;

    program_counter pc0(clk,reset_n,1'b1,PCNext,PC);
    control_unit_full cu(Instr[6:0],Instr[14:12],Instr[30],
      RegWrite,ALUSrc,MemWrite,Branch,ImmSrc,ResultSrc,ALUControl);
    register_file rf(clk,reset_n,RegWrite,Instr[19:15],Instr[24:20],
      Instr[11:7],Result,RD1,RD2);
    extend ext(ImmSrc,Instr[31:7],ImmExt);
    alu alu0(ALUControl,RD1,SrcB,ALUResult,Zero);
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| Register signature | x1=5,x2=7,x3=12,x4=12,x5=0,x6=7,x7=4,x8=7,x9=1 |
| Memory signature | mem[0]=12 |
| Wrong-path addi | x5 is not written |

### Errors requiring deduction or correction

- Checking only one register as the golden result can miss a wrong-path store.

- The loop has no timeout.

- The hex program does not match the reported assembly.

Waveform oracle: The submitted image must allow direct comparison of PC, PCTarget, Branch, Zero, BranchTaken/PCSrc, ALUResult, MemWrite, address, WriteData, and ReadData; distinguish taken from not-taken.

## Experiment 4: FPGA demo

Required conclusion: Map clock/reset/debug without changing the datapath.

### Core answer

- QSF sets FAMILY Cyclone V and DEVICE 5CSXFC6D6F31C6; load constraints/rv32i_labs.sdc.

- CLOCK_50 is the 50 MHz clock; reset_n from KEY must have the correct polarity and should have synchronized deassertion in practical use.

- Debug selects PC/signature through SW and displays it on LEDR/HEX or SignalTap without changing the datapath.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_04/rv32i_fpga_top.v

module rv32i_fpga_top(
    input wire CLOCK_50,
    input wire [3:0] KEY,
    input wire [9:0] SW,
    output wire [9:0] LEDR);
    wire reset_n=KEY[0];
    wire [31:0] PC,Instr,ALUResult,WriteData,ReadData,Result;
    wire MemWrite;
    wire [31:0] selected=SW[0]?Result:PC;
    instruction_memory imem(PC,Instr);
    data_memory dmem(CLOCK_50,MemWrite,ALUResult,WriteData,ReadData);
    single_cycle_core core(CLOCK_50,reset_n,Instr,ReadData,
      PC,ALUResult,WriteData,Result,MemWrite);
    assign LEDR=selected[9:0];
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| Quartus device | 5CSXFC6D6F31C6 |
| Clock report | CLOCK_50 period 20.000 ns |
| Demo | PC/signature is observable and matches simulation |

### Errors requiring deduction or correction

- Reusing the DE2-115 pin map.

- Using a counter bit as the core clock.

- The wrapper reset has polarity different from the testbench.

Waveform oracle: The submitted image must allow direct comparison of PC, PCTarget, Branch, Zero, BranchTaken/PCSrc, ALUResult, MemWrite, address, WriteData, and ReadData; distinguish taken from not-taken.

# 3. Recommended hidden tests

| Test | Stimulus/intent | Oracle |
| --- | --- | --- |
| Word index | sw/lw at offsets 0, 4, and 8. | Three independent words. |
| Backward branch | Count-down loop. | Terminates correctly, without timeout. |
| Wrong-path state | An instruction after a taken branch writes RF or RAM. | No side effects. |
| Illegal opcode | Insert an unsupported word. | All write enables are 0. |

Principle: A hidden test must check functionality and side effects, include a timeout, and not depend on internal hierarchy unless the published interface requires it.

# 4. Answers to oral examination questions

## 1. How do byte addresses and word indices differ?

The ALU creates a byte address. For a 32-bit memory array, the word index drops the two low bits: addr[...:2]. Using the full address as an index places words at 0, 4, 8 instead of 0, 1, 2 and can exceed the depth.

## 2. Why should the target adder be separate from the ALU?

A separate adder computes PC+ImmExt in parallel with the ALU compare/Zero. This makes control flow explicit, avoids changing ALU operands just to compute the target, and eases the transition to EX-stage branch resolution in the pipeline.

## 3. To what extent may the FPGA wrapper change the core?

The wrapper may adapt clock/reset, pins, and observation signals. It must not change ISA semantics, PC step, reset state, memory addressing, or insert logic that makes core results differ from simulation.

Student submission package reference: The main report is the completed student manual exported as DSD_Labxx_MSSV_HoVaTen.pdf. It must contain the student information, RTL code, testbench code, and waveform evidence embedded after each experiment. The package must also include clean rtl/single_cycle/ and sim/single_cycle/ directories, control_table.pdf, trace_one_instruction.pdf, regression.log, and rtl_schematic.png.

# 5. Grading gates and minimum evidence

| Gate | Instructor verification method |
| --- | --- |
| The 9-instruction baseline passes. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Taken, not-taken, and backward beq all behave correctly. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| sw/lw have correct word alignment. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Quartus compiles for 5CSXFC6D6F31C6 and recognizes the 50 MHz clock. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |

# 6. Unified grading rules

| Item | Weight | Answer-key guidance |
| --- | --- | --- |
| Functionally correct RTL | 40% | Pass public and hidden tests; ISA or side-effect errors are severe. |
| Verification | 25% | The test has an oracle, corner cases, a timeout, and a reproducible log. |
| Architecture explanation | 20% | Explain the cause, signal lifetime, and trade-offs; do not merely read the waveform. |
| Interface engineering | 10% | Correct width, names, reset polarity, clock edge, and module contract. |
| Clean submission package | 5% | Runs from a clean folder; no db/output_files/work/WLF. |