INSTRUCTOR ANSWER KEY · INTERNAL · DO NOT DISTRIBUTE TO STUDENTS

# LAB 05 ANSWER KEY
RV32I MULTI-CYCLE AND CONTROL FSM

Reference answers for instruction, grading, and hidden-test construction; run the full regression on the course simulator before official release.

| 05 week | 3 experiments | 4 hidden tests | INTERNAL release level |
| --- | --- | --- | --- |

Language and sources: All solutions use Verilog-2001 (.v). The Week 02/03/04/06/07 code was adapted from the submitted RISC-V codebases; Week 01 and Week 05 were added under the same contract.

Source adjustments: Write a new RV32I FSM because the submitted code is a pipeline, not multi-cycle; keep PC/RF/ALU/memory naming consistent.

Not a golden implementation: If the starter RTL, hex program, or control encoding changes, increment the answer-key version and rerun the full regression before using it for grading.

# 1. Quick answers

| Topic | Required answer/contract |
| --- | --- |
| State paths | R/I=4 cycles; lw=5; sw=4; beq=3 under the reference FSM. |
| Architectural state | PC, RF, memory. |
| Holding registers | IR, OldPC, A, B, ALUOut, MDR; update by enable/state. |
| Safety | Moore outputs default to writes off; illegal opcodes return to IF. |

# 2. Answers by experiment

## Experiment 1: State chart

Required conclusion: Convert the instruction lifetime into an FSM with a safe default.

### Core answer

- R path: IF→ID→EXEC_R→ALUWB→IF.

- addi path: IF→ID→EXEC_I→ALUWB→IF.

- lw: IF→ID→MEMADR→MEMRD→MEMWB→IF; sw replaces MEMRD/MEMWB with MEMWR; beq: IF→ID→BRANCH→IF.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_05/multicycle_controller.v

module multicycle_controller(
    input wire clk, reset_n,
    input wire [6:0] op, input wire [2:0] funct3, input wire funct7_5,
    input wire zero,
    output reg PCWrite, PCSrc, IRWrite, RegWrite, MemWrite,
    output reg AWrite, BWrite, ALUOutWrite, MDRWrite,
    output reg AdrSrc,
    output reg [1:0] ALUSrcA, ALUSrcB, ResultSrc,
    output reg [2:0] ImmSrc, ALUControl,
    output reg [3:0] state);
    localparam IFETCH=4'd0, IDECODE=4'd1, MEMADR=4'd2, MEMRD=4'd3,
               MEMWB=4'd4, MEMWR=4'd5, EXEC_R=4'd6, ALUWB=4'd7,
               EXEC_I=4'd8, BRANCH=4'd9;
    reg [3:0] next_state;
    always @(posedge clk or negedge reset_n)
        if (!reset_n) state<=IFETCH; else state<=next_state;

    always @(*) begin
        next_state=IFETCH;
        case (state)
            IFETCH: next_state=IDECODE;
            IDECODE: case(op)
                7'b0000011,7'b0100011: next_state=MEMADR;
                7'b0110011: next_state=EXEC_R;
                7'b0010011: next_state=EXEC_I;
                7'b1100011: next_state=BRANCH;
                default: next_state=IFETCH;
            endcase
            MEMADR: next_state=(op==7'b0000011)?MEMRD:MEMWR;
            MEMRD: next_state=MEMWB;
            MEMWB,MEMWR,ALUWB,BRANCH: next_state=IFETCH;
            EXEC_R,EXEC_I: next_state=ALUWB;
            default: next_state=IFETCH;
        endcase
    end

    always @(*) begin
        PCWrite=0; PCSrc=0; IRWrite=0; RegWrite=0; MemWrite=0; AdrSrc=0;
        AWrite=0; BWrite=0; ALUOutWrite=0; MDRWrite=0;
        // ALUSrcA: 00=PC, 01=OldPC, 10=A.
        ALUSrcA=2'b00; ALUSrcB=2'b00; ResultSrc=2'b00;
        ImmSrc=3'b000; ALUControl=3'b000;
        case (state)
            IFETCH: begin IRWrite=1; PCWrite=1; ALUSrcB=2'b10; end
            IDECODE: begin
                AWrite=1; BWrite=1; ALUSrcA=2'b01; ALUSrcB=2'b01;
                if (op==7'b1100011) ImmSrc=3'b010;
                else if (op==7'b0100011) ImmSrc=3'b001;
                else ImmSrc=3'b000;
                ALUOutWrite=1;
            end
            MEMADR: begin ALUSrcA=2'b10; ALUSrcB=2'b01; ALUOutWrite=1;
                ImmSrc=(op==7'b0100011)?3'b001:3'b000; end
            MEMRD: begin AdrSrc=1; MDRWrite=1; end
            MEMWB: begin RegWrite=1; ResultSrc=2'b01; end
            MEMWR: begin AdrSrc=1; MemWrite=1; end
            EXEC_R: begin
                ALUSrcA=2'b10; ALUSrcB=2'b00; ALUOutWrite=1;
                case(funct3)
                  3'b000: ALUControl=funct7_5?3'b001:3'b000;
                  3'b111: ALUControl=3'b010;
                  3'b110: ALUControl=3'b011;
                  3'b010: ALUControl=3'b101;
                  default: ALUControl=3'b000;
                endcase
            end
            EXEC_I: begin ALUSrcA=2'b10; ALUSrcB=2'b01; ALUControl=3'b000;
                ALUOutWrite=1; end
            ALUWB: begin RegWrite=1; ResultSrc=2'b00; end
            BRANCH: begin ALUSrcA=2'b10; ALUSrcB=2'b00; ALUControl=3'b001;
                PCSrc=1; if (zero) PCWrite=1; end
            default: begin end
        endcase
    end
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| Illegal opcode in ID | Next state=IF, writes=0 |
| R/addi | RegWrite only in ALUWB |
| sw | MemWrite only in MEMWR |
| lw | RegWrite only in MEMWB |

### Errors requiring deduction or correction

- Using MIPS opcodes in state decode.

- Leaving a write enable asserted across multiple states.

- Having no default recovery.

Waveform oracle: The submitted image must allow direct comparison of state, op, PCWrite/PCSrc, IRWrite, AWrite/BWrite, ALUOutWrite, MDRWrite, RegWrite/MemWrite, ALUSrcA/B, and zero.

## Experiment 2: Datapath reuse

Required conclusion: Use holding registers to reuse the ALU/memory.

### Core answer

- IF captures IR and OldPC while the PC receives PC+4.

- ID captures A/B and may precompute the branch target OldPC+ImmExt into ALUOut.

- MEMADR captures A+ImmExt; MEMRD captures MDR; EXEC_R/I capture ALUOut.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_05/holding_registers.v

module holding_registers(
    input wire clk, reset_n, IRWrite,AWrite,BWrite,ALUOutWrite,MDRWrite,
    input wire [31:0] MemData, PC, RD1, RD2, ALUResult,
    output reg [31:0] IR, OldPC, A, B, ALUOut, MDR);
    always @(posedge clk or negedge reset_n) begin
        if (!reset_n) begin IR<=0; OldPC<=0; A<=0; B<=0; ALUOut<=0; MDR<=0; end
        else begin
            if (IRWrite) begin IR<=MemData; OldPC<=PC; end
            if (AWrite) A<=RD1;
            if (BWrite) B<=RD2;
            if (ALUOutWrite) ALUOut<=ALUResult;
            if (MDRWrite) MDR<=MemData;
        end
    end
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| lw trace | IF,ID,MEMADR,MEMRD,MEMWB |
| OldPC | Hold the instruction PC |
| ALUOut | Hold the address/ALU result until the using state |

### Errors requiring deduction or correction

- Using the incremented PC instead of OldPC when computing the target.

- Updating the holding register every cycle.

- MDR has no explicit enable.

Waveform oracle: The submitted image must allow direct comparison of state, op, PCWrite/PCSrc, IRWrite, AWrite/BWrite, ALUOutWrite, MDRWrite, RegWrite/MemWrite, ALUSrcA/B, and zero.

## Experiment 3: Controller + integration

Required conclusion: Integrate the FSM with the datapath and run the subset regression.

### Core answer

- The state register uses posedge and resets to IF.

- always @(*) sets next_state and all outputs to default 0 before the case.

- The architectural signature must match canonical Week 04 even though the cycle count differs.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_05/tb_multicycle_controller.v

`timescale 1ns/1ps
module tb_multicycle_controller;
    reg clk,reset_n,zero,funct7_5; reg [6:0] op; reg [2:0] funct3; integer errors;
    wire PCWrite,PCSrc,IRWrite,RegWrite,MemWrite,AWrite,BWrite,ALUOutWrite,MDRWrite,AdrSrc;
    wire [1:0] ALUSrcA,ALUSrcB,ResultSrc; wire [2:0] ImmSrc,ALUControl;
    wire [3:0] state;
    multicycle_controller dut(.clk(clk),.reset_n(reset_n),.op(op),.funct3(funct3),
      .funct7_5(funct7_5),.zero(zero),.PCWrite(PCWrite),.PCSrc(PCSrc),
      .IRWrite(IRWrite),
      .RegWrite(RegWrite),.MemWrite(MemWrite),.AWrite(AWrite),.BWrite(BWrite),
      .ALUOutWrite(ALUOutWrite),.MDRWrite(MDRWrite),.AdrSrc(AdrSrc),
      .ALUSrcA(ALUSrcA),.ALUSrcB(ALUSrcB),.ResultSrc(ResultSrc),
      .ImmSrc(ImmSrc),.ALUControl(ALUControl),.state(state));
    always #5 clk=~clk;
    initial begin
        clk=0;reset_n=0;zero=0;funct3=0;funct7_5=0;op=7'b0000011;errors=0;
        #12 reset_n=1;
        @(posedge clk); #1;
        if (state!==4'd1 || ALUSrcA!==2'b01) errors=errors+1;
        @(posedge clk); #1;
        if (state!==4'd2 || ALUSrcA!==2'b10) errors=errors+1;
        repeat(3) @(posedge clk); #1;
        if (state!==4'd0) errors=errors+1;

        reset_n=0; #2; reset_n=1;
        op=7'b1100011; zero=1;
        @(posedge clk); #1;
        if (state!==4'd1 || ImmSrc!==3'b010) errors=errors+1;
        @(posedge clk); #1;
        if (state!==4'd9 || !PCWrite || !PCSrc) errors=errors+1;
        @(posedge clk); #1;
        if (state!==4'd0) errors=errors+1;
        if (errors==0) $display("TEST_PASS"); else $display("TEST_FAIL");
        $finish;
    end
    initial begin #1000;$display("TEST_TIMEOUT");$finish;end
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| Reset | state=S_IF |
| Canonical program | Same RF/RAM signature as Week 04 |
| Write pulses | Each side effect occurs in exactly one state |

### Errors requiring deduction or correction

- A latch caused by a missing default.

- Controller output depends on a combinational opcode during the write-back state.

- Not comparing the signature with single-cycle.

Waveform oracle: The submitted image must allow direct comparison of state, op, PCWrite/PCSrc, IRWrite, AWrite/BWrite, ALUOutWrite, MDRWrite, RegWrite/MemWrite, ALUSrcA/B, and zero.

# 3. Recommended hidden tests

| Test | Stimulus/intent | Oracle |
| --- | --- | --- |
| Write-enable width | Track RegWrite/MemWrite through the full trace. | Each instruction has a pulse only in the correct state. |
| Illegal recovery | Garbage opcode in IR. | Return to IF, no side effect, processor continues. |
| Back-to-back memory | sw followed by lw at the same address. | MDR and ALUOut are not overwritten early. |
| Branch OldPC | Branch after different numbers of cycles. | Target is computed from the branch PC, not the next fetch PC. |

Principle: A hidden test must check functionality and side effects, include a timeout, and not depend on internal hierarchy unless the published interface requires it.

# 4. Answers to oral examination questions

## 1. How does architectural state differ from a holding register?

Architectural state is software-visible state: PC, Register File, and memory. IR, OldPC, A, B, ALUOut, and MDR are temporary microarchitectural state used to complete an instruction; they are not ISA registers.

## 2. How do Moore outputs make the controller safer?

Write enables depend directly on the registered state and do not change with opcode/flag glitches within the same cycle. With writes disabled by default and explicit state recovery, the risk of unintended side effects is reduced.

## 3. Why does multi-cycle not use pipeline registers in the Week 06 sense?

Multi-cycle has one instruction reusing hardware across multiple states; holding registers keep intermediate results. Pipeline registers separate multiple concurrent instructions and carry each instruction’s data/control to the next stage.

Student submission package reference: The main report is the completed student manual exported as DSD_Labxx_MSSV_HoVaTen.pdf. It must contain the student information, RTL code, testbench code, and waveform evidence embedded after each experiment. The package must also include clean rtl/multicycle/ and sim/multicycle/ directories, fsm_state_graph.pdf, control_word_table.pdf, instruction_traces/, and regression.log.

# 5. Grading gates and minimum evidence

| Gate | Instructor verification method |
| --- | --- |
| The FSM does not depend on MIPS opcodes. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Every instruction returns to IF. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Write enables are asserted in only one defined state. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| The architectural signature matches single-cycle. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |

# 6. Unified grading rules

| Item | Weight | Answer-key guidance |
| --- | --- | --- |
| Functionally correct RTL | 40% | Pass public and hidden tests; ISA or side-effect errors are severe. |
| Verification | 25% | The test has an oracle, corner cases, a timeout, and a reproducible log. |
| Architecture explanation | 20% | Explain the cause, signal lifetime, and trade-offs; do not merely read the waveform. |
| Interface engineering | 10% | Correct width, names, reset polarity, clock edge, and module contract. |
| Clean submission package | 5% | Runs from a clean folder; no db/output_files/work/WLF. |