INSTRUCTOR ANSWER KEY · INTERNAL · DO NOT DISTRIBUTE TO STUDENTS

# LAB 06 ANSWER KEY
FIVE-STAGE PIPELINE WITHOUT HAZARDS

Reference answers for instruction, grading, and hidden-test construction; run the full regression on the course simulator before official release.

| 06 week | 4 experiments | 4 hidden tests | INTERNAL release level |
| --- | --- | --- | --- |

Language and sources: All solutions use Verilog-2001 (.v). The Week 02/03/04/06/07 code was adapted from the submitted RISC-V codebases; Week 01 and Week 05 were added under the same contract.

Source adjustments: Reuse reg_if_id/reg_ex_mem and the PC branch adder from the submitted code; move pipeline registers to posedge, represent bubbles as NOPs, and add the no-hazard top.

Not a golden implementation: If the starter RTL, hex program, or control encoding changes, increment the answer-key version and rerun the full regression before using it for grading.

# 1. Quick answers

| Topic | Required answer/contract |
| --- | --- |
| IF/ID | PCD, InstrD, PCPlus4D. |
| ID/EX | Control E, RD1E/RD2E, PCE, Rs1E/Rs2E/RdE, ImmExtE, PCPlus4E. |
| EX/MEM | Control M, ALUResultM, WriteDataM, RdM, PCPlus4M. |
| MEM/WB | Control W, ALUResultW, ReadDataW, RdW, PCPlus4W. |

# 2. Answers by experiment

## Experiment 1: Pipeline-register contracts

Required conclusion: Carry the correct data/control through the correct number of cycles.

### Core answer

- Each pipeline register must carry both data and control for the same instruction.

- Reset/clear creates a bubble by disabling RegWrite/MemWrite/Branch/Jump; data may be zero for clean debugging.

- Reference priority: reset → clear → enable → hold.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_06/pipeline_registers.v

module reg_if_id(
    input wire clk,reset_n,clear,en,
    input wire [31:0] pc_in,instr_in,pc_plus4_in,
    output reg [31:0] pc_out,instr_out,pc_plus4_out);
    always @(posedge clk or negedge reset_n) begin
        if (!reset_n || clear) begin pc_out<=0;instr_out<=32'h00000013;pc_plus4_out<=0;end
        else if (en) begin pc_out<=pc_in;instr_out<=instr_in;pc_plus4_out<=pc_plus4_in;end
    end
endmodule

module reg_id_ex(
    input wire clk,reset_n,clear,
    input wire RegWriteD,ALUSrcD,MemWriteD,BranchD,
    input wire [1:0] ResultSrcD,input wire [2:0] ALUControlD,
    input wire [31:0] RD1D,RD2D,PCD,ImmExtD,PCPlus4D,
    input wire [4:0] Rs1D,Rs2D,RdD,
    output reg RegWriteE,ALUSrcE,MemWriteE,BranchE,
    output reg [1:0] ResultSrcE,output reg [2:0] ALUControlE,
    output reg [31:0] RD1E,RD2E,PCE,ImmExtE,PCPlus4E,
    output reg [4:0] Rs1E,Rs2E,RdE);
    always @(posedge clk or negedge reset_n) begin
        if (!reset_n || clear) begin
          RegWriteE<=0;ALUSrcE<=0;MemWriteE<=0;BranchE<=0;ResultSrcE<=0;
          ALUControlE<=0;RD1E<=0;RD2E<=0;PCE<=0;ImmExtE<=0;PCPlus4E<=0;
          Rs1E<=0;Rs2E<=0;RdE<=0;
        end else begin
          RegWriteE<=RegWriteD;ALUSrcE<=ALUSrcD;MemWriteE<=MemWriteD;
          BranchE<=BranchD;ResultSrcE<=ResultSrcD;ALUControlE<=ALUControlD;
          RD1E<=RD1D;RD2E<=RD2D;PCE<=PCD;ImmExtE<=ImmExtD;PCPlus4E<=PCPlus4D;
          Rs1E<=Rs1D;Rs2E<=Rs2D;RdE<=RdD;
        end
    end
endmodule

module reg_ex_mem(
    input wire clk,reset_n,
    input wire RegWriteE,MemWriteE,input wire [1:0] ResultSrcE,
    input wire [31:0] ALUResultE,WriteDataE,PCPlus4E,input wire [4:0] RdE,
    output reg RegWriteM,MemWriteM,output reg [1:0] ResultSrcM,
    output reg [31:0] ALUResultM,WriteDataM,PCPlus4M,output reg [4:0] RdM);
    always @(posedge clk or negedge reset_n) begin
        if (!reset_n) begin RegWriteM<=0;MemWriteM<=0;ResultSrcM<=0;
          ALUResultM<=0;WriteDataM<=0;PCPlus4M<=0;RdM<=0;end
        else begin RegWriteM<=RegWriteE;MemWriteM<=MemWriteE;ResultSrcM<=ResultSrcE;
          ALUResultM<=ALUResultE;WriteDataM<=WriteDataE;PCPlus4M<=PCPlus4E;RdM<=RdE;end
    end
endmodule

module reg_mem_wb(
    input wire clk,reset_n,input wire RegWriteM,input wire [1:0] ResultSrcM,
    input wire [31:0] ReadDataM,ALUResultM,PCPlus4M,input wire [4:0] RdM,
    output reg RegWriteW,output reg [1:0] ResultSrcW,
    output reg [31:0] ReadDataW,ALUResultW,PCPlus4W,output reg [4:0] RdW);
    always @(posedge clk or negedge reset_n) begin
        if (!reset_n) begin RegWriteW<=0;ResultSrcW<=0;ReadDataW<=0;
          ALUResultW<=0;PCPlus4W<=0;RdW<=0;end
        else begin RegWriteW<=RegWriteM;ResultSrcW<=ResultSrcM;ReadDataW<=ReadDataM;
          ALUResultW<=ALUResultM;PCPlus4W<=PCPlus4M;RdW<=RdM;end
    end
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| hold | All output registers hold |
| clear | Control side effect=0 |
| normal | Data/control advance by one stage together |

### Errors requiring deduction or correction

- Omitting Rd/ResultSrc/PCPlus4.

- Clearing only data while keeping write enables asserted.

- Using fields on a different clock edge.

Waveform oracle: The submitted image must allow direct comparison of PCF/PCD/PCE, InstrD, the pipeline registers, and E/M/W control; trace one instruction through F–D–E–M–W and the NOPs.

## Experiment 2: Five-stage integration

Required conclusion: Integrate the pipeline before adding forwarding/stall.

### Core answer

- Latency from F to W is five stages; once full, ideal throughput is one instruction per cycle.

- A trace using PC or an instruction tag must show the same instruction moving F→D→E→M→W.

- Week 06 locks forwarding=00, stall=0, flush=0 to isolate alignment.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_06/pipeline_nohaz_core.v

module pipeline_nohaz_core(
    input wire clk, reset_n,
    output wire [31:0] PCF, ResultW,
    output wire PCSrcE);
    wire [31:0] PCNext,PCPlus4F,InstrF,PCD,InstrD,PCPlus4D;
    wire [31:0] RD1D,RD2D,ImmExtD,PCE,RD1E,RD2E,ImmExtE,PCPlus4E;
    wire [31:0] SrcBE,ALUResultE,PCTargetE,ALUResultM,WriteDataM;
    wire [31:0] ReadDataM,PCPlus4M,ReadDataW,ALUResultW,PCPlus4W;
    wire [4:0] Rs1E,Rs2E,RdE,RdM,RdW;
    wire RegWriteD,ALUSrcD,MemWriteD,BranchD;
    wire RegWriteE,ALUSrcE,MemWriteE,BranchE;
    wire RegWriteM,MemWriteM,RegWriteW,ZeroE;
    wire [1:0] ImmSrcD,ResultSrcD,ResultSrcE,ResultSrcM,ResultSrcW;
    wire [2:0] ALUControlD,ALUControlE;

    assign PCPlus4F=PCF+32'd4;
    assign PCTargetE=PCE+ImmExtE;
    assign PCSrcE=BranchE&ZeroE;
    assign PCNext=PCSrcE?PCTargetE:PCPlus4F;
    program_counter pc0(clk,reset_n,1'b1,PCNext,PCF);
    instruction_memory imem(PCF,InstrF);
    reg_if_id fd(clk,reset_n,PCSrcE,1'b1,PCF,InstrF,PCPlus4F,
      PCD,InstrD,PCPlus4D);

    control_unit_full cu(InstrD[6:0],InstrD[14:12],InstrD[30],
      RegWriteD,ALUSrcD,MemWriteD,BranchD,ImmSrcD,ResultSrcD,ALUControlD);
    register_file rf(clk,reset_n,RegWriteW,InstrD[19:15],InstrD[24:20],
      RdW,ResultW,RD1D,RD2D);
    extend ext(ImmSrcD,InstrD[31:7],ImmExtD);
    reg_id_ex de(clk,reset_n,PCSrcE,RegWriteD,ALUSrcD,MemWriteD,BranchD,
      ResultSrcD,ALUControlD,RD1D,RD2D,PCD,ImmExtD,PCPlus4D,
      InstrD[19:15],InstrD[24:20],InstrD[11:7],RegWriteE,ALUSrcE,
      MemWriteE,BranchE,ResultSrcE,ALUControlE,RD1E,RD2E,PCE,ImmExtE,
      PCPlus4E,Rs1E,Rs2E,RdE);

    assign SrcBE=ALUSrcE?ImmExtE:RD2E;
    alu alu0(ALUControlE,RD1E,SrcBE,ALUResultE,ZeroE);
    reg_ex_mem em(clk,reset_n,RegWriteE,MemWriteE,ResultSrcE,
      ALUResultE,RD2E,PCPlus4E,RdE,RegWriteM,MemWriteM,ResultSrcM,
      ALUResultM,WriteDataM,PCPlus4M,RdM);
    data_memory dmem(clk,MemWriteM,ALUResultM,WriteDataM,ReadDataM);
    reg_mem_wb mw(clk,reset_n,RegWriteM,ResultSrcM,ReadDataM,ALUResultM,
      PCPlus4M,RdM,RegWriteW,ResultSrcW,ReadDataW,ALUResultW,PCPlus4W,RdW);
    assign ResultW=(ResultSrcW==2'b01)?ReadDataW:
                   (ResultSrcW==2'b10)?PCPlus4W:ALUResultW;
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| One independent add | Write-back at W with the correct rd/data |
| One sw | Address and store data meet at M |
| One lw | ReadDataM travels with Rd/ResultSrc to W |

### Errors requiring deduction or correction

- WriteDataM is taken directly from RD2D.

- RegWriteW is advanced or delayed relative to RdW.

- Using negedge to hide alignment errors.

Waveform oracle: The submitted image must allow direct comparison of PCF/PCD/PCE, InstrD, the pipeline registers, and E/M/W control; trace one instruction through F–D–E–M–W and the NOPs.

## Experiment 3: NOP-scheduled regression

Required conclusion: Confirm the basic pipeline core when hazards are eliminated by software.

### Core answer

- With posedge Register File write and posedge ID/EX capture, and no WB→D bypass, the consumer must enter D after the write-back edge: use 3 NOPs between producer and consumer.

- If the course defines same-cycle write-before-read or adds a WB→D bypass, 2 NOPs are sufficient; state the contract and test the chosen option.

- Memory/branch must also be scheduled to avoid RAW/control hazards.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_06/nop_scheduled_program.v

module nop_scheduled_program(input wire [31:0] PC,output reg [31:0] Instr);
    always @(*) begin
        case (PC[6:2])
          0:Instr=32'h00500093; // addi x1,x0,5
          1:Instr=32'h00000013; 2:Instr=32'h00000013; 3:Instr=32'h00000013;
          4:Instr=32'h00700113; // addi x2,x0,7
          5:Instr=32'h00000013; 6:Instr=32'h00000013; 7:Instr=32'h00000013;
          8:Instr=32'h002081B3; // add x3,x1,x2
          default:Instr=32'h00000013;
        endcase
    end
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| Independent program | PASS |
| 3-NOP RAW schedule | PASS under the default contract |
| Remove NOPs | May FAIL; the failure is explained as a hazard |

### Errors requiring deduction or correction

- Claiming 2 NOPs without stating the RF timing contract.

- Accidentally allowing the compiler/assembler to reorder instructions.

- Using a hazardous test to conclude that alignment is wrong.

Waveform oracle: The submitted image must allow direct comparison of PCF/PCD/PCE, InstrD, the pipeline registers, and E/M/W control; trace one instruction through F–D–E–M–W and the NOPs.

## Experiment 4: Branch resolve at EX

Required conclusion: Compute the target with a separate adder and redirect when the instruction is in E.

### Core answer

- PCTargetE=PCE+ImmExtE uses a separate adder; the ALU still creates ZeroE from compare.

- BranchTakenE=BranchE&ZeroE; PCSrcE=BranchTakenE|JumpE.

- Because there is no flush yet, the two instructions after a branch must be NOPs in the Week 06 program.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_06/pc_branch_adder.v

module pc_branch_adder(
    input wire [31:0] PCE,ImmExtE,
    output wire [31:0] PCTargetE);
    assign PCTargetE=PCE+ImmExtE;
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| beq taken | PCF receives PCTargetE |
| beq not-taken | Sequential PC |
| BranchE=0,ZeroE=1 | PCSrcE=0 |

### Errors requiring deduction or correction

- Using ALUResultE as the target while the ALU is comparing.

- Computing from PCF/PCD instead of PCE.

- Inserting only one NOP after branch resolution at E.

Waveform oracle: The submitted image must allow direct comparison of PCF/PCD/PCE, InstrD, the pipeline registers, and E/M/W control; trace one instruction through F–D–E–M–W and the NOPs.

# 3. Recommended hidden tests

| Test | Stimulus/intent | Oracle |
| --- | --- | --- |
| Control/data tag | Tag the instruction and track it to W. | No stage misalignment. |
| Store alignment | Two independent stores with different address/data. | Do not mix one instruction’s address with another instruction’s data. |
| NOP distance | Producer/consumer with 0–3 NOPs. | Only configurations meeting the contract must PASS. |
| Branch slots | Taken branch with a side-effecting instruction after it. | The Week 06 program must place two NOPs, so there is no side effect. |

Principle: A hidden test must check functionality and side effects, include a timeout, and not depend on internal hierarchy unless the published interface requires it.

# 4. Answers to oral examination questions

## 1. Which control signals must reach M/W?

M needs RegWriteM, ResultSrcM, and MemWriteM. W needs RegWriteW and ResultSrcW. Associated data includes ALUResult, WriteData, ReadData, Rd, and PCPlus4 according to the write-back source.

## 2. Why do NOPs isolate alignment errors?

NOPs remove RAW/control interactions, so if the signature is still wrong, the main cause is likely lifetime or incorrect data/control alignment across pipeline registers. Add forwarding/stall only after alignment is correct.

## 3. How many wrong-path instructions does a branch at EX create?

When the branch is in E, two younger instructions are in D and F. Because Week 06 has no flush, a safe program needs two NOP delay slots for a taken branch; Week 07 removes both with FlushE and FlushD.

Student submission package reference: The main report is the completed student manual exported as DSD_Labxx_MSSV_HoVaTen.pdf. It must contain the student information, RTL code, testbench code, and waveform evidence embedded after each experiment. The package must also include clean rtl/pipeline_nohaz/ and sim/pipeline_nohaz/ directories, pipeline_contract.xlsx or PDF, trace_5_stage.pdf, nop_schedule.S, and regression.log.

# 5. Grading gates and minimum evidence

| Gate | Instructor verification method |
| --- | --- |
| The four pipeline registers pass hold/clear tests. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| The NOP-scheduled baseline reaches the signature. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Branch target/decision is located at EX. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| The waveform trace shows the same instruction through F/D/E/M/W. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |

# 6. Unified grading rules

| Item | Weight | Answer-key guidance |
| --- | --- | --- |
| Functionally correct RTL | 40% | Pass public and hidden tests; ISA or side-effect errors are severe. |
| Verification | 25% | The test has an oracle, corner cases, a timeout, and a reproducible log. |
| Architecture explanation | 20% | Explain the cause, signal lifetime, and trade-offs; do not merely read the waveform. |
| Interface engineering | 10% | Correct width, names, reset polarity, clock edge, and module contract. |
| Clean submission package | 5% | Runs from a clean folder; no db/output_files/work/WLF. |