INSTRUCTOR ANSWER KEY · INTERNAL · DO NOT DISTRIBUTE TO STUDENTS

# LAB 07 ANSWER KEY
HAZARD-AWARE PIPELINE AND FPGA CAPSTONE

Reference answers for instruction, grading, and hidden-test construction; run the full regression on the course simulator before official release.

| 07 week | 4 experiments | 5 hidden tests | INTERNAL release level |
| --- | --- | --- | --- |

Language and sources: All solutions use Verilog-2001 (.v). The Week 02/03/04/06/07 code was adapted from the submitted RISC-V codebases; Week 01 and Week 05 were added under the same contract.

Source adjustments: Reuse Hazard_Unit.v/Pipelined_RISCV.v; fix M-before-W priority, RdE!=0, UseRs1D/UseRs2D, store-data forwarding, and the integrated top.

Not a golden implementation: If the starter RTL, hex program, or control encoding changes, increment the answer-key version and rerun the full regression before using it for grading.

# 1. Quick answers

| Topic | Required answer/contract |
| --- | --- |
| Forward priority | M→E (10) has priority over W→E (01); Rd=0 or RegWrite=0 does not forward. |
| Store data | Forward operand B before the ALUSrc mux; WriteDataE comes from forwarded B. |
| Load-use | Stall F/D for one cycle and flush E; RdE must be nonzero. |
| Control hazard | PCSrcE redirects the PC and flushes both D/E to remove two younger instructions. |

# 2. Answers by experiment

## Experiment 1: EX forwarding

Required conclusion: Resolve RAW hazards when the result is in M or W.

### Core answer

- ForwardAE/BE default to 00; an M match selects 10, and W is checked for 01 only when M does not match.

- Source 10 is ALUResultM; source 01 is ResultW.

- WriteDataE must be forwarded_B, while SrcBE selects between forwarded_B and ImmExtE.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_07/forwarding_unit.v

module forwarding_unit(
    input wire [4:0] Rs1E,Rs2E,RdM,RdW,
    input wire RegWriteM,RegWriteW,
    output reg [1:0] ForwardAE,ForwardBE);
    always @(*) begin
        ForwardAE=2'b00; ForwardBE=2'b00;
        if (RegWriteM && RdM!=0 && RdM==Rs1E) ForwardAE=2'b10;
        else if (RegWriteW && RdW!=0 && RdW==Rs1E) ForwardAE=2'b01;
        if (RegWriteM && RdM!=0 && RdM==Rs2E) ForwardBE=2'b10;
        else if (RegWriteW && RdW!=0 && RdW==Rs2E) ForwardBE=2'b01;
    end
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| M and W both match | Select M |
| RdM/RdW=0 | 00 |
| RegWrite=0 | 00 |
| ALU→store data | RAM receives the new value |

### Errors requiring deduction or correction

- W overrides M because of two independent if statements.

- Forwarding the load address from M as if it were load data.

- Forwarding SrcBE only after the mux, so sw still writes old data.

Waveform oracle: The submitted image must allow direct comparison of ForwardAE/BE, SrcAE/SrcBE, StallF/StallD, FlushD/FlushE, PCSrcE, RdE/RdM/RdW, and RegWrite/ResultSrc; the correct bubble/flush behavior must be visible.

## Experiment 2: Load-use stall

Required conclusion: Hold F/D and insert one bubble into E for exactly one cycle.

### Core answer

- Load-use is valid only when the instruction in E is a load, RdE!=0, and D actually reads that register.

- The baseline may compare both Rs1D/Rs2D; a false stall only reduces performance. An optimized version uses UseRs1D/UseRs2D from the decoder.

- StallF=StallD=lwStall and FlushE includes lwStall. After one cycle, the consumer remains in D and the load moves to W to forward ResultW.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_07/hazard_unit.v

module hazard_unit(
    input wire [4:0] Rs1D,Rs2D,Rs1E,Rs2E,RdE,RdM,RdW,
    input wire RegWriteM,RegWriteW,ResultSrcE0,PCSrcE,
    input wire UseRs1D,UseRs2D,
    output wire [1:0] ForwardAE,ForwardBE,
    output wire StallF,StallD,FlushE,FlushD);
    wire lwStall;
    forwarding_unit fw(Rs1E,Rs2E,RdM,RdW,RegWriteM,RegWriteW,
                       ForwardAE,ForwardBE);
    assign lwStall=ResultSrcE0 && (RdE!=5'd0) &&
      ((UseRs1D && Rs1D==RdE)||(UseRs2D && Rs2D==RdE));
    assign StallF=lwStall;
    assign StallD=lwStall;
    assign FlushD=PCSrcE;
    assign FlushE=lwStall|PCSrcE;
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| lw→ALU | Exactly 1 bubble |
| lw→sw data | Exactly 1 bubble and correct store |
| lw rd=x0 | No stall |
| No dependency | No stall |

### Errors requiring deduction or correction

- Omitting RdE!=0.

- Stalling E instead of inserting a bubble.

- Stalling for two cycles.

- Treating a false stall as a functional error even though the baseline contract permits it.

Waveform oracle: The submitted image must allow direct comparison of ForwardAE/BE, SrcAE/SrcBE, StallF/StallD, FlushD/FlushE, PCSrcE, RdE/RdM/RdW, and RegWrite/ResultSrc; the correct bubble/flush behavior must be visible.

## Experiment 3: Control hazard

Required conclusion: Flush wrong-path instructions when the PC redirects at EX.

### Core answer

- When PCSrcE=1, the PC receives PCTargetE at the next edge; the instructions in D and F are both wrong-path.

- FlushE removes instruction D before it enters E; FlushD removes instruction F before it enters D.

- Wrong-path RegWrite and MemWrite must be 0 in the side-effect stages.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_07/hazard_datapath_connections.v

module hazard_datapath_connections(
    input wire [31:0] RD1E,RD2E,ResultW,ALUResultM,ImmExtE,
    input wire [1:0] ForwardAE,ForwardBE,input wire ALUSrcE,
    output reg [31:0] SrcAE,ForwardedB,
    output wire [31:0] WriteDataE,SrcBE);
    always @(*) begin
        case(ForwardAE)
          2'b01:SrcAE=ResultW; 2'b10:SrcAE=ALUResultM; default:SrcAE=RD1E;
        endcase
        case(ForwardBE)
          2'b01:ForwardedB=ResultW; 2'b10:ForwardedB=ALUResultM;
          default:ForwardedB=RD2E;
        endcase
    end
    assign WriteDataE=ForwardedB;
    assign SrcBE=ALUSrcE?ImmExtE:ForwardedB;
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| taken beq | Two bubbles are inserted, PC=PCTargetE |
| not-taken | No flush |
| wrong-path sw/addi | No RAM/RF write |

### Errors requiring deduction or correction

- Flushing only FlushD or only FlushE.

- Flushing data without disabling write controls.

- PC redirect is correct but the wrong-path store still commits.

Waveform oracle: The submitted image must allow direct comparison of ForwardAE/BE, SrcAE/SrcBE, StallF/StallD, FlushD/FlushE, PCSrcE, RdE/RdM/RdW, and RegWrite/ResultSrc; the correct bubble/flush behavior must be visible.

## Experiment 4: Capstone regression + FPGA

Required conclusion: Freeze the contract and create reproducible evidence.

### Core answer

- Run Hazard Unit tests first, followed by integration and canonical architectural regression without NOPs.

- Golden RF/RAM must match single-cycle/multi-cycle; only the cycle count differs.

- The FPGA wrapper observes PC/signature/hazard state, QSF loads the SDC, and the timing report has no unconstrained clock.

### Verilog-2001 solution code

Reference file: INSTRUCTOR/REFERENCE_RTL/Week_07/pipelined_rv32i_hazard.v

module pipelined_rv32i_hazard(
    input wire clk, reset_n,
    output wire [31:0] PCF,ResultW,
    output wire [1:0] ForwardAE,ForwardBE,
    output wire StallF,StallD,FlushD,FlushE);
    wire [31:0] PCNext,PCPlus4F,InstrF,PCD,InstrD,PCPlus4D;
    wire [31:0] RD1D,RD2D,ImmExtD,PCE,RD1E,RD2E,ImmExtE,PCPlus4E;
    wire [31:0] SrcAE,ForwardedB,SrcBE,WriteDataE,ALUResultE,PCTargetE;
    wire [31:0] ALUResultM,WriteDataM,ReadDataM,PCPlus4M;
    wire [31:0] ReadDataW,ALUResultW,PCPlus4W;
    wire [4:0] Rs1E,Rs2E,RdE,RdM,RdW;
    wire RegWriteD,ALUSrcD,MemWriteD,BranchD;
    wire RegWriteE,ALUSrcE,MemWriteE,BranchE;
    wire RegWriteM,MemWriteM,RegWriteW,ZeroE,PCSrcE;
    wire UseRs1D,UseRs2D;
    wire [1:0] ImmSrcD,ResultSrcD,ResultSrcE,ResultSrcM,ResultSrcW;
    wire [2:0] ALUControlD,ALUControlE;

    assign PCPlus4F=PCF+32'd4;
    assign PCTargetE=PCE+ImmExtE;
    assign PCSrcE=BranchE&ZeroE;
    assign PCNext=PCSrcE?PCTargetE:PCPlus4F;
    program_counter pc0(clk,reset_n,~StallF,PCNext,PCF);
    instruction_memory imem(PCF,InstrF);
    reg_if_id fd(clk,reset_n,FlushD,~StallD,PCF,InstrF,PCPlus4F,
      PCD,InstrD,PCPlus4D);

    control_unit_full cu(InstrD[6:0],InstrD[14:12],InstrD[30],
      RegWriteD,ALUSrcD,MemWriteD,BranchD,ImmSrcD,ResultSrcD,ALUControlD);
    register_file rf(clk,reset_n,RegWriteW,InstrD[19:15],InstrD[24:20],
      RdW,ResultW,RD1D,RD2D);
    extend ext(ImmSrcD,InstrD[31:7],ImmExtD);
    reg_id_ex de(clk,reset_n,FlushE,RegWriteD,ALUSrcD,MemWriteD,BranchD,
      ResultSrcD,ALUControlD,RD1D,RD2D,PCD,ImmExtD,PCPlus4D,
      InstrD[19:15],InstrD[24:20],InstrD[11:7],RegWriteE,ALUSrcE,
      MemWriteE,BranchE,ResultSrcE,ALUControlE,RD1E,RD2E,PCE,ImmExtE,
      PCPlus4E,Rs1E,Rs2E,RdE);

    hazard_datapath_connections fwd_data(RD1E,RD2E,ResultW,ALUResultM,
      ImmExtE,ForwardAE,ForwardBE,ALUSrcE,SrcAE,ForwardedB,WriteDataE,SrcBE);
    alu alu0(ALUControlE,SrcAE,SrcBE,ALUResultE,ZeroE);
    reg_ex_mem em(clk,reset_n,RegWriteE,MemWriteE,ResultSrcE,
      ALUResultE,WriteDataE,PCPlus4E,RdE,RegWriteM,MemWriteM,ResultSrcM,
      ALUResultM,WriteDataM,PCPlus4M,RdM);
    data_memory dmem(clk,MemWriteM,ALUResultM,WriteDataM,ReadDataM);
    reg_mem_wb mw(clk,reset_n,RegWriteM,ResultSrcM,ReadDataM,ALUResultM,
      PCPlus4M,RdM,RegWriteW,ResultSrcW,ReadDataW,ALUResultW,PCPlus4W,RdW);
    assign ResultW=(ResultSrcW==2'b01)?ReadDataW:
                   (ResultSrcW==2'b10)?PCPlus4W:ALUResultW;

    assign UseRs1D=(InstrD[6:0]==7'b0110011)||(InstrD[6:0]==7'b0010011)||
                   (InstrD[6:0]==7'b0000011)||(InstrD[6:0]==7'b0100011)||
                   (InstrD[6:0]==7'b1100011);
    assign UseRs2D=(InstrD[6:0]==7'b0110011)||(InstrD[6:0]==7'b0100011)||
                   (InstrD[6:0]==7'b1100011);
    hazard_unit hz(InstrD[19:15],InstrD[24:20],Rs1E,Rs2E,RdE,RdM,RdW,
      RegWriteM,RegWriteW,ResultSrcE[0],PCSrcE,UseRs1D,UseRs2D,
      ForwardAE,ForwardBE,StallF,StallD,FlushE,FlushD);
endmodule

### Expected result

| Scenario | Correct result |
| --- | --- |
| No-NOP canonical program | Signature matches Week 04/05 |
| ALU RAW | 0 stall |
| Load-use | 1 stall |
| Taken branch | 2 younger instructions flushed |

### Errors requiring deduction or correction

- Running only a demo waveform.

- Making the golden result depend on internal cycles instead of architectural state.

- Submitting Quartus db/work/WLF artifacts.

Waveform oracle: The submitted image must allow direct comparison of ForwardAE/BE, SrcAE/SrcBE, StallF/StallD, FlushD/FlushE, PCSrcE, RdE/RdM/RdW, and RegWrite/ResultSrc; the correct bubble/flush behavior must be visible.

# 3. Recommended hidden tests

| Test | Stimulus/intent | Oracle |
| --- | --- | --- |
| M/W double match | Two consecutive producers write the same rd, followed immediately by the consumer. | Forward from M. |
| Load to store | lw x5,0(x0); sw x5,4(x0). | One stall, mem[1]=loaded value. |
| x0 load | lw x0,0(x0) followed by an independent instruction. | No stall/forward for x0. |
| Wrong-path store | Taken beq followed by sw and addi in the two younger slots. | No memory/register side effect. |
| PCSrcE with pending dependency | Create a control case with flush and stall close together. | PC redirect wins functionally; D/E are both flushed correctly. |

Principle: A hidden test must check functionality and side effects, include a timeout, and not depend on internal hierarchy unless the published interface requires it.

# 4. Answers to oral examination questions

## 1. When M and W both match, which source is selected?

Select M because it is the newer value. W may be selected only when M does not match the same operand.

## 2. Why cannot load-use be resolved with the current forwarding alone?

When the consumer enters E, the load is only in M and ALUResultM is only the address; memory data is not yet available at the M→E forwarding source. A one-cycle stall moves the load to W so the consumer receives ResultW.

## 3. How does FlushE differ from StallD in effect?

StallD holds IF/ID so instruction D can retry. FlushE turns the content entering ID/EX into a no-side-effect bubble. Load-use requires holding F/D and flushing E; branch redirect requires flushing D and E.

Student submission package reference: The main report is the completed student manual exported as DSD_Labxx_MSSV_HoVaTen.pdf. It must contain the student information, RTL code, testbench code, and waveform evidence embedded after each experiment. The package must also include clean rtl/pipeline_hazard/ and sim/unit+integration+regression/ directories, programs/, constraints/, logs/, waveforms/, and fpga_evidence/.

# 5. Grading gates and minimum evidence

| Gate | Instructor verification method |
| --- | --- |
| ALU RAW runs without NOPs. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Load-use creates exactly one bubble. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Taken branch flushes the correct wrong-path instructions. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |
| Regression runs from a clean directory and FPGA compile meets timing. | PASS when the log/automated test proves it; the waveform is explanatory evidence only. |

# 6. Unified grading rules

| Item | Weight | Answer-key guidance |
| --- | --- | --- |
| Functionally correct RTL | 40% | Pass public and hidden tests; ISA or side-effect errors are severe. |
| Verification | 25% | The test has an oracle, corner cases, a timeout, and a reproducible log. |
| Architecture explanation | 20% | Explain the cause, signal lifetime, and trade-offs; do not merely read the waveform. |
| Interface engineering | 10% | Correct width, names, reset polarity, clock edge, and module contract. |
| Clean submission package | 5% | Runs from a clean folder; no db/output_files/work/WLF. |