# REFERENCED CIRCUIT AND VERILOG CODES

Refer to the tutorial “Quartus II Introduction Using Verilog Design” to create the first project in Quartus II and implement a simple circuit on the DE2-115 kit.

# II.1 EXPERIMENT 1: BASIC LOGIC GATES

## Referenced Verilog code in three modeling styles: dataflow, behavioral, and gate-level:

## Gate-level (Structural Model)  :

module allgate_GL ( a, b, yand,yor,ynot,ynand,ynor,yxor,yxnor );

input a,b;

output yand, yor, ynot, ynand, ynor, yxor, yxnor;

and    G1(yand,a,b);		// AND Operation

or      G2(yor,a, b);		// OR Operation

not    G3(ynot,a) ;		// NOT Operation

nand G4 (ynand,a,b);		// NAND Operation

nor    G5(ynor,a,b);		//NOR Operation

xor    G6(yxor,a,b);		//XOR Operation

xnor  G7(yxnor,a,b);		//XNOR Operation

endmodule				// END of the module

Write the Top-level verilog module to implement the allgate_GL module in DE2-FPGA Kit

# II.2 LAB EXPERIMENT 2 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT THE HALF ADDER CIRCUIT:

## Dataflow model:

module half_adder_dataflow(input a, b, output s, Cout);

assign s = a ^ b;

assign Cout = a & b;

endmodule

## Behavior Model :

module half_adder_behavior(sum,carry,a,b );

output sum,carry;

input a,b;

reg sum,carry;

always @(*)

begin

sum   <= a ^ b;

carry <= a&b  ;

end

endmodule

## Gate-level (Structural Model)  :

module half_adder_structural(input a, b, output s, Cout);

xor G1(s,a,b);

and G2(Cout,a,b);

endmodule

# II.3 EXPERIMENT 3: WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT THE FULL ADDER CIRCUIT:

## Dataflow model:

module full_adder_DF(input a, b, cin, output S, Cout);

assign S = a ^ b ^ cin;

assign Cout = (a & b) | (b & cin) | (a & cin);

endmodule

## Behavior Model using case :

module full_adder_BH1 (input wire A, B, Cin, output reg S, output reg Cout);

always @(*)

begin

case ({A, B, Cin})

3'b000: begin S = 0; Cout = 0; end

3'b001: begin S = 1; Cout = 0; end

3'b010: begin S = 1; Cout = 0; end

3'b011: begin S = 0; Cout = 1; end

3'b100: begin S = 1; Cout = 0; end

3'b101: begin S = 0; Cout = 1; end

3'b110: begin S = 0; Cout = 1; end

3'b111: begin S = 1; Cout = 1; end

endcase

end

## Behavior Model using if else :

module full_adder( A, B, Cin, S, Cout);

input wire A, B, Cin;

output reg S, Cout;

always @(*)

begin

if(A==0 && B==0 && Cin==0)

begin

S=0;

Cout=0;

end

else if(A==0 && B==0 && Cin==1)

begin

S=1;

Cout=0;

end

else if(A==0 && B==1 && Cin==0)

begin

S=1;

Cout=0;

end

else if(A==0 && B==1 && Cin==1)

begin

S=0;

Cout=1;

end

else if(A==1 && B==0 && Cin==0)

begin

S=1;

Cout=0;

end

else if(A==1 && B==0 && Cin==1)

begin

S=0;

Cout=1;

end

else if(A==1 && B==1 && Cin==0)

begin

S=0;

Cout=1;

end

else if(A==1 && B==1 && Cin==1)

begin

S=1;

Cout=1;

end

end

endmodule

endmodule

module full_adder_BH(a,b,c,sum,carry);

output sum,carry;

input  a,b,c;

reg    sum,carry;

always @(*)

begin

sum   <= a^ b^c;

carry <=(a&b) | (b&c) | (c&a);

end

endmodule

## Gate-level (Structural Model)  :

module full_adder_STRU(A, B, Cin, Sum, Carry);

output Sum,Carry;

input A,B,Cin;

wire x,y,z;

xor g1(x,A,B);

xor g2(Sum,x,Cin);

and g3(y,x,Cin);

and g4(z,A,B);

or  g5(Carry,x,y);

endmodule

# II.4 EXPERIMENT 4 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT THE  RIPPLE CARRY ADDER CIRCUIT:

## Structural Model:

module full_adder_STRU(A, B, Cin, Sum, Carry);

output Sum,Carry;

input A,B,Cin;

wire x,y,z;

xor g1(x,A,B);

xor g2(Sum,x,Cin);

and g3(y,x,Cin);

and g4(z,A,B);

or  g5(Carry,x,y);

endmodule

module four_bit_adder_STRU(cin,a,b,s,cout);

input [3:0] a,b;

input cin;

output [3:0] s;

output cout;

wire [2:0] w_carry;

full_adder_STRU  C1(a[0],b[0],cin,s[0], w_carry[0]);

full_adder_STRU  C2(a[1],b[1], w_carry[0],s[1], w_carry[1]);

full_adder_STRU  C3(a[2],b[2], w_carry[1],s[2], w_carry[2]);

full_adder_STRU  C4(a[3],b[3], w_carry[2],s[3], cout);

endmodule

## Dataflow model:

module Four_bit_Adder_DF(A,B,Cin,Sum,Cout);

input [3:0] A,B;

input Cin;

output wire [3:0]Sum;

output wire Cout;

wire [4:0]temp;

assign temp=A+B+Cin;

assign Sum=temp[3:0];

assign Cout=temp[4];

endmodule

# II.5 EXPERIMENT 5 :WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT  2:1 MULTIPLEXER CIRCUIT:

## Dataflow model:

module mux21_data_flow(i0,i1,sel,y);

input i0,i1,sel;

output y;

assign y =(i0&(~sel))|(i1&sel);

endmodule

## Behavior Model :

module mux21_Behavioural (i0,i1,sel,y);

input i0,i1,sel;

output y;

reg y;

always@(*)

begin

if (sel == 1'b0) y = i0;

else y = i1;

end

endmodule

## Structural Model:

module mux21_structural(i0,i1,sel,y);

input i0,i1,sel;

output y;

wire net1,net2,net3;

not g1(net1,sel);

and g2(net2,i1,sel);

and g3(net3,i0,net1);

or g4(y,net3,net2);

endmodule

# II.6 EXPERIMENT 6 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT  4:1 MULTIPLEXER CIRCUIT:

## Structural Model:

module mux41_structural (i0,i1,i2,i3,s0,s1,y);

input i0,i1,i2,i3,s0,s1;

output y;

wire n_s0,n_s1, a0,a1,a2,a3;

not g0(n_s0,s0);

not g1(n_s1,s1);

and g2(a0,i0,n_s0,n_s1);

and g3(a1,i1,n_s1,s0);

and g4(a2,i2,s1,n_s0);

and g5(a3,i3,s1,s0);

or   g6(y,a0,a1,a2,a3);

endmodule

## Dataflow model:

module mux41_df (i0,i1,i2,i3,s0,s1,y);

input i0,i1,i2,i3,s0,s1;

output y;

assign y= i0&(~s1)&(~s0) | i1 &(~s1)&s0  | i2&s1&(~s0) | i3&s1&s0;

endmodule

module mux41beh_v1(in,s,y );

output y ;

input [3:0] in ;

input [1:0] s ;

reg y;

always @(*)

begin

if (s == 2'b00)

y <= in[0];

else if (s == 2'b01)

y <= in[1];

else if (s == 2'b10)

y <= in[2];

else

y <= in[3];

end

endmodule

## Behavior Model :

module mux41beh_v2(in,s,y );

output y ;

input [3:0]in ;

input [1:0]s ;

reg y;

always @(*)

begin

case (s)

2'b00: y <= in[0];

2'b01: y <= in[1];

2'b10: y <= in[2];

2'b11: y <= in[3];

endcase

end

endmodule

# II.7 EXPERIMENT 7 : WRITE VERILOG HDL CODES TO SIMULATE AND IMPLEMENT  16:1 MULTIPLEXER CIRCUIT USING STRUCTURAL MODEL FROM FIVE 4:1 MULTIPLEXER MODULE