LAB 04 · WEEK 04

# SINGLE-CYCLE MEMORY, BRANCH AND FPGA

Complete lw, sw, and beq; check side effects and bring the baseline core to Cyclone V.

| 180′ in-lab | 3–5h pre/post lab | 4 experiments | 14% course weight |
| --- | --- | --- | --- |

### Student information

| Full name: __________________________________________ | Student ID: ______________________________ |
| --- | --- |
| Course: Digital System Design (DSD) | Group/Class: __________________________ |
| Submission date: __________________________________________ | Instructor: __________________________ |

Weekly outcome: Students complete the 9-instruction baseline, verify branch/memory, and create an observable FPGA demo without changing the core.

HDL language: Use Verilog-2001 during the foundation stage: .v files, wire/reg, always @(*) and always @(posedge clk...). SystemVerilog is introduced only after students have mastered Verilog.

### Evidence requirements after each experiment

Required: Immediately after each experiment, students must submit the RTL code, testbench, and one waveform screenshot captured from ModelSim or Questa. The screenshot must show the test name, time scale, clock/reset, stimulus signals, the output under test, and the PASS/FAIL line if printed by the testbench.

vlog -2001 rtl/*.v sim/tb_*.v
vsim work.tb_<experiment_name>
add wave -r /*
run -all

Capture procedure: After the signals are stable, select Zoom Full or the cycle range being graded, then use the ModelSim/Questa Capture/Export Image function. Insert the image in the waveform area immediately after the experiment; do not submit only a WLF file.

# 1. Pre-lab preparation

- Lab 03 regression PASS.

- Understand S/B immediates and word addressing.

- Have standard QSF/SDC constraints for 5CSXFC6D6F31C6.

# 2. Architecture used

Figure 4.1 · Datapath/module area used in Week 04.

# 3. Module contract

| Module/block | Main inputs | Main outputs |
| --- | --- | --- |
| data_memory | clk, we, addr, write_data | read_data |
| branch_logic | Branch, Zero | PCSrc |
| fpga_wrapper | CLOCK_50, KEY, SW | LEDR/HEX debug |
| single_cycle_core | clk, reset_n | memory write + signature |

# 4. Mandatory common contract

| Item | Convention |
| --- | --- |
| HDL language | Verilog-2001 (.v): wire/reg, always @(*), always @(posedge clk); do not use SystemVerilog syntax yet |
| Required ISA | RV32I subset: add, sub, and, or, slt, addi, lw, sw, beq |
| Extension | jal is enabled only when JumpE, ResultSrc=10, and the corresponding test is complete |
| Clock/reset | posedge clk; reset_n active-low; no gated clock, and do not use negedge to avoid a hazard |
| Address | PC increments by 4; instruction/data memory uses word addressing through addr[...:2] |
| Signal naming | Use F/D/E/M/W stage suffixes; state widths explicitly in the interface |
| Verification | Every test must determine PASS/FAIL automatically and include a timeout; submit a ModelSim/Questa waveform image after each experiment |

# 5. Core code skeleton — Verilog-2001

wire BranchTaken;
assign BranchTaken = Branch & Zero;
assign PCNext = BranchTaken ? PCTarget : PCPlus4;
// jal remains an optional extension, not part of the baseline gate.

# 6. Lab procedure

## Experiment 1: Load/store

Aim: Preserve alignment and write-enable behavior.

### Procedure

- Implement asynchronous read and synchronous write according to the lab contract.

- Test sw→lw at the same and different addresses.

- Check that MemWrite=0 leaves memory unchanged.

### Checkpoints

- Addresses use a word index.

- Writes occur only at the clock edge.

- ReadData is stable for a valid address.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_01_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_01.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_01_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_01_run.log |

### RTL code entry area

// EXPERIMENT 01 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 01 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

## Experiment 2: Branch on the EX-equivalent path

Aim: In the single-cycle design, decide the PC using compare and an independent target adder.

### Procedure

- Compute PCTarget=PC+ImmExt.

- Generate BranchTaken=Branch&Zero.

- Test taken, not-taken, and backward branches.

### Checkpoints

- Taken selects the target.

- Not-taken selects PC+4.

- The instruction after a branch must not execute incorrectly.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_02_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_02.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_02_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_02_run.log |

### RTL code entry area

// EXPERIMENT 02 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 02 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

## Experiment 3: Full baseline regression

Aim: Run the 9 instructions in a program with a loop, memory, and a signature.

### Procedure

- Integrate lw/sw/beq.

- Add a timeout and an illegal-opcode safety test.

- Check the final register and memory signatures.

### Checkpoints

- No unexpected side effects.

- The signature matches the golden value.

- Regression runs from a clean checkout.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_03_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_03.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_03_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_03_run.log |

### RTL code entry area

// EXPERIMENT 03 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 03 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

## Experiment 4: FPGA demo

Aim: Map clock/reset/debug without changing the datapath.

### Procedure

- Use CLOCK_50 and reset_n from KEY.

- Select PC/signature with SW.

- Observe the output on LEDR/HEX or SignalTap.

### Checkpoints

- QSF targets the correct device.

- QSF loads the SDC file.

- Do not create a derived clock with logic.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_04_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_04.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_04_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_04_run.log |

### RTL code entry area

// EXPERIMENT 04 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 04 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

# 7. Standard simulation procedure

- Compile all sources from a clean directory; classify every warning.

- Run the new module unit test before the integration test.

- Run directed corner cases, followed by a regression with a timeout.

- Open the waveform only for a failing test or a required checkpoint.

- Save a log containing the test name, seed, ending cycle, and PASS/FAIL result.

# 8. Acceptance gate

- PASS — The 9-instruction baseline passes.

- PASS — beq taken/not-taken/backward all behave correctly.

- PASS — sw/lw have correct word alignment.

- PASS — Quartus compiles for 5CSXFC6D6F31C6 and recognizes the 50 MHz clock.

# 9. Submission package

Required main report: DSD_Labxx_MSSV_HoVaTen.pdf

Export the completed student manual as this PDF. It must include the student information, RTL code, testbench code, and waveform evidence inserted in the designated sections after each experiment.

The submission package must contain:

- rtl/single_cycle_full/

- sim/regression/

- programs/baseline.hex

- constraints/

- quartus_summary.pdf

- fpga_evidence/

Do not submit: db/, incremental_db/, output_files/, work/, temporary transcripts, large WLF files, backups, or solution references.

# 10. Weekly rubric

| Item | Weight | Minimum condition |
| --- | --- | --- |
| Functionally correct RTL | 40% | Public and hidden tests PASS; no timeout. |
| Verification | 25% | Self-checking, corner cases, and a reproducible log. |
| Architecture explanation | 20% | Correctly trace signal lifetime and control causes. |
| Interface engineering | 10% | Correct names, widths, polarity, and clock/reset contract. |
| Clean submission package | 5% | Contains no tool-generated output; the script runs from a clean folder. |

# 11. Oral examination questions

- How do byte addresses and word indices differ?

- Why should the target adder be separate from the ALU?

- To what extent may the FPGA wrapper change the core?

# Reference

RISC-V Unprivileged ISA Specification, RV32I Base Integer Instruction Set: https://docs.riscv.org/reference/isa/unpriv/rv32.html