LAB 05 · WEEK 05

# RV32I MULTI-CYCLE AND CONTROL FSM

Reuse datapath blocks over time to understand state sequencing before moving to the pipeline.

| 180′ in-lab | 3–5h pre/post lab | 3 experiments | 15% course weight |
| --- | --- | --- | --- |

### Student information

| Full name: __________________________________________ | Student ID: ______________________________ |
| --- | --- |
| Course: Digital System Design (DSD) | Group/Class: __________________________ |
| Submission date: __________________________________________ | Instructor: __________________________ |

Weekly outcome: Students design an IF/ID/EX/MEM/WB FSM for the required subset and prove that the control outputs are correct in each state.

HDL language: Use Verilog-2001 during the foundation stage: .v files, wire/reg, always @(*) and always @(posedge clk...). SystemVerilog is introduced only after students have mastered Verilog.

### Evidence requirements after each experiment

Required: Immediately after each experiment, students must submit the RTL code, testbench, and one waveform screenshot captured from ModelSim or Questa. The screenshot must show the test name, time scale, clock/reset, stimulus signals, the output under test, and the PASS/FAIL line if printed by the testbench.

vlog -2001 rtl/*.v sim/tb_*.v
vsim work.tb_<experiment_name>
add wave -r /*
run -all

Capture procedure: After the signals are stable, select Zoom Full or the cycle range being graded, then use the ModelSim/Questa Capture/Export Image function. Insert the image in the waveform area immediately after the experiment; do not submit only a WLF file.

# 1. Pre-lab preparation

- Single-cycle baseline PASS.

- Draw the state transitions for add/addi/lw/sw/beq.

- Distinguish architectural state from temporary state.

# 2. Architecture used

Figure 5.1 · Datapath/module area used in Week 05.

Figure note: The figure only divides functional regions taken from the pipeline datapath. Students must draw the physical multi-cycle datapath themselves; do not treat the pipeline registers in the figure as multi-cycle holding registers.

# 3. Module contract

| Module/block | Main inputs | Main outputs |
| --- | --- | --- |
| mc_datapath | control word + memory/RF data | opcode/funct + ALU flags |
| mc_controller | clk, reset_n, op, funct3, zero | PCWrite, IRWrite, RegWrite, MemWrite, selects |
| holding_registers | IR, OldPC, A, B, ALUOut, MDR | stored values |
| multicycle_core | clk, reset_n | debug state + signature |

# 4. Mandatory common contract

| Item | Convention |
| --- | --- |
| HDL language | Verilog-2001 (.v): wire/reg, always @(*), always @(posedge clk); do not use SystemVerilog syntax yet |
| Required ISA | RV32I subset: add, sub, and, or, slt, addi, lw, sw, beq |
| Extension | jal is enabled only when JumpE, ResultSrc=10, and the corresponding test is complete |
| Clock/reset | posedge clk; reset_n active-low; no gated clock, and do not use negedge to avoid a hazard |
| Address | PC increments by 4; instruction/data memory uses word addressing through addr[...:2] |
| Signal naming | Use F/D/E/M/W stage suffixes; state widths explicitly in the interface |
| Verification | Every test must determine PASS/FAIL automatically and include a timeout; submit a ModelSim/Questa waveform image after each experiment |

# 5. Core code skeleton — Verilog-2001

localparam [3:0] S_IF=4'd0, S_ID=4'd1, S_MEMADR=4'd2,
  S_MEMRD=4'd3, S_MEMWB=4'd4, S_MEMWR=4'd5,
  S_EXEC_R=4'd6, S_ALUWB=4'd7, S_EXEC_I=4'd8, S_BRANCH=4'd9;
reg [3:0] state, next_state;
// TODO: state register + next-state/Moore outputs with safe defaults

# 6. Lab procedure

## Experiment 1: State chart

Aim: Convert the instruction lifetime into an FSM with a safe default.

### Procedure

- Draw the state graph.

- Create the Moore-output table for each state.

- Identify shared states and instruction-specific states.

### Checkpoints

- Every path returns to IF.

- MemWrite/RegWrite are asserted only in the correct state.

- An illegal opcode safely returns to IF.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_01_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_01.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_01_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_01_run.log |

### RTL code entry area

// EXPERIMENT 01 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 01 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

## Experiment 2: Datapath reuse

Aim: Use holding registers to reuse the ALU/memory.

### Procedure

- Implement IR, A, B, ALUOut, and MDR.

- Determine ALUSrcA/B and ResultSrc for each state.

- Trace one lw through all five steps.

### Checkpoints

- A holding register updates only when enabled.

- OldPC is not changed before the target is computed.

- ALUOut retains its value across the required states.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_02_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_02.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_02_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_02_run.log |

### RTL code entry area

// EXPERIMENT 02 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 02 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

## Experiment 3: Controller + integration

Aim: Integrate the FSM with the datapath and run the subset regression.

### Procedure

- Implement the state register on posedge.

- Implement combinational next-state/output logic.

- Run add/addi/lw/sw/beq.

### Checkpoints

- No latch in the controller.

- The cycle count matches the state graph.

- The signature matches the single-cycle design.

### Submission evidence immediately after the experiment

| Required item | Content to enter/attach | Suggested filename |
| --- | --- | --- |
| RTL code | All modules/blocks for the experiment, using correct Verilog-2001. | exp_03_rtl.v |
| Testbench | Self-checking testbench with an oracle, error counter, and timeout. | tb_exp_03.v |
| Waveform | ModelSim/Questa image showing clock/reset, inputs, outputs, and the checking moment. | exp_03_waveform.png |
| Result log | Compile command, simulator, runtime, and TEST_PASS/TEST_FAIL. | exp_03_run.log |

### RTL code entry area

// EXPERIMENT 03 — STUDENT RTL CODE (Verilog-2001)
// Paste the student module code here or record the submitted filename.

### Testbench entry area

// EXPERIMENT 03 — STUDENT TESTBENCH
// Paste the self-checking testbench code here or record the submitted filename.

Evidence insertion area: Paste the RTL code, testbench code, and waveform image in the designated locations when completing the report. If separate files are submitted, record the exact corresponding paths.

# 7. Standard simulation procedure

- Compile all sources from a clean directory; classify every warning.

- Run the new module unit test before the integration test.

- Run directed corner cases, followed by a regression with a timeout.

- Open the waveform only for a failing test or a required checkpoint.

- Save a log containing the test name, seed, ending cycle, and PASS/FAIL result.

# 8. Acceptance gate

- PASS — The FSM does not depend on MIPS opcodes.

- PASS — Every instruction returns to IF.

- PASS — Write enables are asserted in only one defined state.

- PASS — The architectural signature matches single-cycle.

# 9. Submission package

Required main report: DSD_Labxx_MSSV_HoVaTen.pdf

Export the completed student manual as this PDF. It must include the student information, RTL code, testbench code, and waveform evidence inserted in the designated sections after each experiment.

The submission package must contain:

- rtl/multicycle/

- sim/multicycle/

- fsm_state_graph.pdf

- control_word_table.pdf

- instruction_traces/

- regression.log

Do not submit: db/, incremental_db/, output_files/, work/, temporary transcripts, large WLF files, backups, or solution references.

# 10. Weekly rubric

| Item | Weight | Minimum condition |
| --- | --- | --- |
| Functionally correct RTL | 40% | Public and hidden tests PASS; no timeout. |
| Verification | 25% | Self-checking, corner cases, and a reproducible log. |
| Architecture explanation | 20% | Correctly trace signal lifetime and control causes. |
| Interface engineering | 10% | Correct names, widths, polarity, and clock/reset contract. |
| Clean submission package | 5% | Contains no tool-generated output; the script runs from a clean folder. |

# 11. Oral examination questions

- How does architectural state differ from a holding register?

- How do Moore outputs make the controller safer?

- Why does multi-cycle not use pipeline registers in the Week 06 sense?

# Reference

RISC-V Unprivileged ISA Specification, RV32I Base Integer Instruction Set: https://docs.riscv.org/reference/isa/unpriv/rv32.html